# 6. API Endpoints

## 6.1 GET /api/v1/tags/list

List all tags with metadata used by clients for display and write validation.

Auth: required

Response fields per tag:

| Field | Description |
| --- | --- |
| tag_name | Tag key, e.g. Tank01_TempSV |
| device_address | PLC address, e.g. D4000 or D1002.8 |
| data_type | Bool, Word, or Float |
| read_write | R = read-only, W = writable |
| multiplier | Engineering scale factor (read: raw×mult, write: value÷mult) |
| description | Human-readable text; falls back to tag_name if empty |
| low_limit | Present only when set in TagsMap.xlsx |
| high_limit | Present only when set in TagsMap.xlsx (HighLimit/UpLimit) |

Example (PowerShell):

$key = (Get-Content dev_api_key.txt).Trim()
Invoke-RestMethod "http://127.0.0.1:8000/api/v1/tags/list" -Headers @{"X-API-Key"=$key}

Example response (abbreviated):

{
  "tags": [
    {
      "tag_name": "Tank01_TempSV",
      "device_address": "D4000",
      "data_type": "Word",
      "read_write": "W",
      "multiplier": 0.1,
      "description": "Tank01 Temperature Set Value",
      "low_limit": 0.0,
      "high_limit": 999.9
    }
  ]
}

## 6.2 GET /api/v1/tags/read

Return the most recent poll cycle from the database, including engineering values for every tag. Description and limits are not included here.

Auth: required

Common response fields:

| Field | Description |
| --- | --- |
| id | Database poll row id |
| timestamp | ISO timestamp of the poll |
| poll_count | Cyclic poll slot (1..MAX_POLL_CYCLES) |
| tags_ok | Number of successful tag reads |
| tags_failed | Number of failed tag reads |
| <tag_name> | Engineering value for each tag (null if read failed) |

Example:

Invoke-RestMethod "http://127.0.0.1:8000/api/v1/tags/read" -Headers @{"X-API-Key"=$key}

## 6.3 POST /api/v1/tags/write

Write one engineering value to a writable tag (ReadWrite=W). The API acquires the shared PLC lock, connects to the PLC, writes, then releases the lock.

Auth: required

Request body:

{
  "tag_name": "Tank01_TempSV",
  "value": 25.0
}

Value rules:

Bool: 0 or 1 only

Word / Float: engineering units (after Multiplier scaling to PLC raw)

If LowLimit/HighLimit exist in TagsMap, value must be within range

Example:

$body = @{ tag_name = "Tank01_TempSV"; value = 25.0 } | ConvertTo-Json
Invoke-RestMethod "http://127.0.0.1:8000/api/v1/tags/write" `
  -Method Post -Headers @{"X-API-Key"=$key; "Content-Type"="application/json"} `
  -Body $body

Success response fields include tag_name, device_address, data_type, read_write, multiplier, value (engineering), and written=true.

## 6.4 POST /api/v1/tags/write/batch

Write multiple writable tags in one PLC session (1–100 items). Duplicate tag names in the same batch are rejected.

Request body:

{
  "writes": [
    { "tag_name": "Tank01_TempSV", "value": 25.0 },
    { "tag_name": "Tank01_HeaterOn", "value": 1 }
  ]
}

Example:

$body = @{
  writes = @(
    @{ tag_name = "Tank01_TempSV"; value = 25.0 },
    @{ tag_name = "Tank01_HeaterOn"; value = 1 }
  )
} | ConvertTo-Json -Depth 4
Invoke-RestMethod "http://127.0.0.1:8000/api/v1/tags/write/batch" `
  -Method Post -Headers @{"X-API-Key"=$key; "Content-Type"="application/json"} `
  -Body $body

# 7. TagsMap Concepts Affecting the API

## 7.1 Multiplier

Engineering value = PLC raw × Multiplier on read. On write, PLC raw = round(engineering ÷ Multiplier) for Word tags. Example: Multiplier 0.1, write 25.0 → PLC register 250; read 250 → API 25.0.

## 7.2 Data types

| DataType | Behavior |
| --- | --- |
| Bool | Bit devices, or bit-of-word addresses like D1002.8 |
| Word | 16-bit register; integer after scaling |
| Float | IEEE-754 REAL over 2 consecutive D registers |

## 7.3 Limits

LowLimit and HighLimit in TagsMap.xlsx are enforced on write using engineering units. Out-of-range values return HTTP 400. Blank limits mean no bound for that side.

## 7.4 Descriptions

Returned by /tags/list. Tank tags are prefixed with the tank column name (e.g. “Tank01 Temperature Set Value”). Empty Description falls back to tag_name.

# 8. Tag Catalog (from TagsMap.xlsx)

Complete list of tags currently loaded from TagsMap.xlsx (361 tags: 141 writable, 220 read-only). Regenerate this manual after TagsMap changes to keep the list current.

| Tag Name | Address | Type | R/W | Mult | Description | Low | High |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alarm01 | M100 | Bool | R | 1.0 | The power Supply phases are swapped |  |  |
| Alarm02 | M101 | Bool | R | 1.0 | Emergency Stop |  |  |
| Alarm03 | M102 | Bool | R | 1.0 | HMI Emergency Stop |  |  |
| Alarm04 | M103 | Bool | R | 1.0 | 12# Emergency Stop |  |  |
| Alarm05 | M104 | Bool | R | 1.0 | 2# Emergency Stop |  |  |
| Alarm06 | M105 | Bool | R | 1.0 | 9# Emergency Stop |  |  |
| Alarm07 | M106 | Bool | R | 1.0 | spare |  |  |
| Alarm08 | M107 | Bool | R | 1.0 | spare |  |  |
| Alarm09 | M108 | Bool | R | 1.0 | spare |  |  |
| Alarm10 | M109 | Bool | R | 1.0 | spare |  |  |
| Alarm11 | M110 | Bool | R | 1.0 | spare |  |  |
| Alarm12 | M111 | Bool | R | 1.0 | spare |  |  |
| Alarm13 | M112 | Bool | R | 1.0 | spare |  |  |
| Alarm14 | M113 | Bool | R | 1.0 | spare |  |  |
| Alarm15 | M114 | Bool | R | 1.0 | spare |  |  |
| Alarm16 | M115 | Bool | R | 1.0 | spare |  |  |
| Alarm17 | M116 | Bool | R | 1.0 | spare |  |  |
| Alarm18 | M117 | Bool | R | 1.0 | spare |  |  |
| Alarm19 | M118 | Bool | R | 1.0 | spare |  |  |
| Alarm20 | M119 | Bool | R | 1.0 | spare |  |  |
| Alarm21 | M120 | Bool | R | 1.0 | 1# Filter Pump Overload |  |  |
| Alarm22 | M121 | Bool | R | 1.0 | 3# Filter Pump Overload |  |  |
| Alarm23 | M122 | Bool | R | 1.0 | 5# Filter Pump Overload |  |  |
| Alarm24 | M123 | Bool | R | 1.0 | 5# Swing Pump Overload |  |  |
| Alarm25 | M124 | Bool | R | 1.0 | 7# Filter Pump Overload |  |  |
| Alarm26 | M125 | Bool | R | 1.0 | 10# Filter Pump Overload |  |  |
| Alarm27 | M126 | Bool | R | 1.0 | 11# Filter Pump Overload |  |  |
| Alarm28 | M127 | Bool | R | 1.0 | 14# Filter Pump Overload |  |  |
| Alarm29 | M128 | Bool | R | 1.0 | 16# Filter Pump Overload |  |  |
| Alarm30 | M129 | Bool | R | 1.0 | 1# Heater |  |  |
| Alarm31 | M130 | Bool | R | 1.0 | 3# Heater |  |  |
| Alarm32 | M131 | Bool | R | 1.0 | 5# Heater |  |  |
| Alarm33 | M132 | Bool | R | 1.0 | 7# Heater |  |  |
| Alarm34 | M133 | Bool | R | 1.0 | 10# Heater |  |  |
| Alarm35 | M134 | Bool | R | 1.0 | 13# Heater |  |  |
| Alarm36 | M135 | Bool | R | 1.0 | 14# Heater |  |  |
| Alarm37 | M136 | Bool | R | 1.0 | 16# Heater |  |  |
| Alarm38 | M137 | Bool | R | 1.0 | Front Water Leak |  |  |
| Alarm39 | M138 | Bool | R | 1.0 | Back Water Leak |  |  |
| Alarm40 | M139 | Bool | R | 1.0 | spare |  |  |
| Alarm41 | M140 | Bool | R | 1.0 | spare |  |  |
| Alarm42 | M141 | Bool | R | 1.0 | spare |  |  |
| Alarm43 | M142 | Bool | R | 1.0 | spare |  |  |
| Alarm44 | M143 | Bool | R | 1.0 | spare |  |  |
| Alarm45 | M144 | Bool | R | 1.0 | spare |  |  |
| Alarm46 | M145 | Bool | R | 1.0 | spare |  |  |
| Alarm47 | M146 | Bool | R | 1.0 | spare |  |  |
| Alarm48 | M147 | Bool | R | 1.0 | spare |  |  |
| Alarm49 | M148 | Bool | R | 1.0 | spare |  |  |
| Alarm50 | M149 | Bool | R | 1.0 | spare |  |  |
| Alarm51 | M150 | Bool | R | 1.0 | 1# tank level low |  |  |
| Alarm52 | M151 | Bool | R | 1.0 | 1# buffer tank level high |  |  |
| Alarm53 | M152 | Bool | R | 1.0 | 1# Buffer tank level low |  |  |
| Alarm54 | M153 | Bool | R | 1.0 | 3# tank level low |  |  |
| Alarm55 | M154 | Bool | R | 1.0 | 3# buffer tank level high |  |  |
| Alarm56 | M155 | Bool | R | 1.0 | 3# Buffer tank level low |  |  |
| Alarm57 | M156 | Bool | R | 1.0 | 5# tank level low |  |  |
| Alarm58 | M157 | Bool | R | 1.0 | 5# buffer tank level high |  |  |
| Alarm59 | M158 | Bool | R | 1.0 | 5# Buffer tank level low |  |  |
| Alarm60 | M159 | Bool | R | 1.0 | 7# tank level low |  |  |
| Alarm61 | M160 | Bool | R | 1.0 | 7# buffer tank level high |  |  |
| Alarm62 | M161 | Bool | R | 1.0 | 7# Buffer tank level low |  |  |
| Alarm63 | M162 | Bool | R | 1.0 | 10# tank level low |  |  |
| Alarm64 | M163 | Bool | R | 1.0 | 10# buffer tank level high |  |  |
| Alarm65 | M164 | Bool | R | 1.0 | 10# Buffer tank level low |  |  |
| Alarm66 | M165 | Bool | R | 1.0 | 11# tank level low |  |  |
| Alarm67 | M166 | Bool | R | 1.0 | 13# tank level high |  |  |
| Alarm68 | M167 | Bool | R | 1.0 | 13# tank level low |  |  |
| Alarm69 | M168 | Bool | R | 1.0 | 14# tank level low |  |  |
| Alarm70 | M169 | Bool | R | 1.0 | 14# buffer tank level high |  |  |
| Alarm71 | M170 | Bool | R | 1.0 | 14# Buffer tank level low |  |  |
| Alarm72 | M171 | Bool | R | 1.0 | 16# tank level low |  |  |
| Alarm73 | M172 | Bool | R | 1.0 | 16# buffer tank level high |  |  |
| Alarm74 | M173 | Bool | R | 1.0 | 16# Buffer tank level low |  |  |
| Alarm75 | M174 | Bool | R | 1.0 | spare |  |  |
| Alarm76 | M175 | Bool | R | 1.0 | spare |  |  |
| Alarm77 | M176 | Bool | R | 1.0 | spare |  |  |
| Alarm78 | M177 | Bool | R | 1.0 | spare |  |  |
| Alarm79 | M178 | Bool | R | 1.0 | spare |  |  |
| Alarm80 | M179 | Bool | R | 1.0 | spare |  |  |
| Local | L3 | Bool | R | 1.0 | Local (Manual Mode) |  |  |
| MainInlet | L190 | Bool | W | 1.0 | Main Inlet Always On |  |  |
| MainInletStatus | Y5 | Bool | R | 1.0 | Main Inlet Status |  |  |
| Remote | L4 | Bool | R | 1.0 | Remote (API Mode) |  |  |
| Tank01A_USSV | D1044 | Word | W | 0.1 | Tank01A Ultrasonic Generator Current (A) Set Value | 0.0 | 8.0 |
| Tank01B_USSV | D1048 | Word | W | 0.1 | Tank01B Ultrasonic Generator Current (A) Set Value | 0.0 | 8.0 |
| Tank01_DoorCloseRemote | M301 | Bool | W | 1.0 | Tank01 Door Remote Close |  |  |
| Tank01_DoorClosed | D1000.5 | Bool | R | 1.0 | Tank01 Door Actually Closed |  |  |
| Tank01_DoorOpenRemote | M300 | Bool | W | 1.0 | Tank01 Door Remote Open |  |  |
| Tank01_DoorOpened | D1000.4 | Bool | R | 1.0 | Tank01 Door Actually Opened |  |  |
| Tank01_FillWaterAuto | L200 | Bool | W | 1.0 | Tank01 Fill Water Auto |  |  |
| Tank01_FillWaterOn | L100 | Bool | W | 1.0 | Tank01 Fill Water always On |  |  |
| Tank01_FillWaterStatus | D1002.8 | Bool | R | 1.0 | Tank01 Fill Water Valve Current Status |  |  |
| Tank01_FilterRemote | M320 | Bool | W | 1.0 | Tank01 Filter Remote On |  |  |
| Tank01_FilterStatus | D1002.1 | Bool | R | 1.0 | Tank01 Filter Pump Status |  |  |
| Tank01_HaveBasket | M601 | Bool | R | 1.0 | Tank01 The Tank has Basket |  |  |
| Tank01_HeaterRemote | M321 | Bool | W | 1.0 | Tank01 Heater Remote On |  |  |
| Tank01_HeaterStatus | D1002.2 | Bool | R | 1.0 | Tank01 Heater Status |  |  |
| Tank01_RemoteStart | M323 | Bool | W | 1.0 | Tank01 Remote Start |  |  |
| Tank01_TempAdj | D4010 | Word | W | 0.1 | Tank01 Temperature +- Adjust | 0.0 | 999.9 |
| Tank01_TempPV | D4000 | Word | R | 0.1 | Tank01 Temperature Present Value |  |  |
| Tank01_TempSV | D4020 | Word | W | 0.1 | Tank01 Temperature Set Value | 0.0 | 999.9 |
| Tank01_TimePV | CN101 | Word | R | 1.0 | Tank01 Time Present Value |  |  |
| Tank01_TimeSV | D4051 | Word | W | 1.0 | Tank01 Time Set Value | 0.0 | 9999.0 |
| Tank01_TimeUp | CS101 | Bool | R | 1.0 | Tank01 Time Up |  |  |
| Tank01_USRemote | M322 | Bool | W | 1.0 | Tank01 Ultrasonic Remote On |  |  |
| Tank01_USStatus | Y4 | Bool | R | 1.0 | Tank01 Ultrasonic Status |  |  |
| Tank01_WaterFlow | D1004 | Float | R | 1.0 | Tank01 Water Flow Meter Value (LPM) |  |  |
| Tank02_AirAgitationAuto | L202 | Bool | W | 1.0 | Tank02 Air Agitation Auto |  |  |
| Tank02_AirAgitationOn | L102 | Bool | W | 1.0 | Tank02 Air Agitation Always On |  |  |
| Tank02_AirAgitationStatus | D1002.8 | Bool | R | 1.0 | Tank02 Air Agitation Status |  |  |
| Tank02_FillWaterAuto | L201 | Bool | W | 1.0 | Tank02 Fill Water Auto |  |  |
| Tank02_FillWaterOn | L101 | Bool | W | 1.0 | Tank02 Fill Water always On |  |  |
| Tank02_FillWaterStatus | D1002.C | Bool | R | 1.0 | Tank02 Fill Water Valve Current Status |  |  |
| Tank02_HaveBasket | M602 | Bool | R | 1.0 | Tank02 The Tank has Basket |  |  |
| Tank02_RemoteStart | M324 | Bool | W | 1.0 | Tank02 Remote Start |  |  |
| Tank02_TimePV | CN102 | Word | R | 1.0 | Tank02 Time Present Value |  |  |
| Tank02_TimeSV | D4052 | Word | W | 1.0 | Tank02 Time Set Value | 0.0 | 9999.0 |
| Tank02_TimeUp | CS102 | Bool | R | 1.0 | Tank02 Time Up |  |  |
| Tank02_WaterFlow | D1006 | Word | R | 0.1 | Tank02 Water Flow Meter Value (LPM) |  |  |
| Tank03_DoorCloseRemote | M303 | Bool | W | 1.0 | Tank03 Door Remote Close |  |  |
| Tank03_DoorClosed | D1010.5 | Bool | R | 1.0 | Tank03 Door Actually Closed |  |  |
| Tank03_DoorOpenRemote | M302 | Bool | W | 1.0 | Tank03 Door Remote Open |  |  |
| Tank03_DoorOpened | D1010.4 | Bool | R | 1.0 | Tank03 Door Actually Opened |  |  |
| Tank03_FillWaterAuto | L220 | Bool | W | 1.0 | Tank03 Fill Water Auto |  |  |
| Tank03_FillWaterOn | L110 | Bool | W | 1.0 | Tank03 Fill Water always On |  |  |
| Tank03_FillWaterStatus | D1012.8 | Bool | R | 1.0 | Tank03 Fill Water Valve Current Status |  |  |
| Tank03_FilterRemote | M332 | Bool | W | 1.0 | Tank03 Filter Remote On |  |  |
| Tank03_FilterStatus | D1012.1 | Bool | R | 1.0 | Tank03 Filter Pump Status |  |  |
| Tank03_HaveBasket | M603 | Bool | R | 1.0 | Tank03 The Tank has Basket |  |  |
| Tank03_HeaterRemote | M333 | Bool | W | 1.0 | Tank03 Heater Remote On |  |  |
| Tank03_HeaterStatus | D1012.2 | Bool | R | 1.0 | Tank03 Heater Status |  |  |
| Tank03_RemoteStart | M330 | Bool | W | 1.0 | Tank03 Remote Start |  |  |
| Tank03_SwingRemote | M334 | Bool | W | 1.0 | Tank03 Swing Remote On |  |  |
| Tank03_SwingSV | D1008 | Word | W | 1.0 | Tank03 Swing Speed Set Value | 0.0 | 2000.0 |
| Tank03_SwingStatus | D1012.4 | Bool | R | 1.0 | Tank03 Swing Status |  |  |
| Tank03_TempAdj | D4011 | Word | W | 0.1 | Tank03 Temperature +- Adjust | 0.0 | 999.9 |
| Tank03_TempPV | D4001 | Word | R | 0.1 | Tank03 Temperature Present Value |  |  |
| Tank03_TempSV | D4021 | Word | W | 0.1 | Tank03 Temperature Set Value | 0.0 | 999.9 |
| Tank03_TimePV | CN103 | Word | R | 1.0 | Tank03 Time Present Value |  |  |
| Tank03_TimeSV | D4053 | Word | W | 1.0 | Tank03 Time Set Value | 0.0 | 9999.0 |
| Tank03_TimeUp | CS103 | Bool | R | 1.0 | Tank03 Time Up |  |  |
| Tank03_WaterFlow | D1014 | Float | R | 1.0 | Tank03 Water Flow Meter Value (LPM) |  |  |
| Tank04_AirAgitationAuto | L212 | Bool | W | 1.0 | Tank04 Air Agitation Auto |  |  |
| Tank04_AirAgitationOn | L112 | Bool | W | 1.0 | Tank04 Air Agitation Always On |  |  |
| Tank04_AirAgitationStatus | D1012.8 | Bool | R | 1.0 | Tank04 Air Agitation Status |  |  |
| Tank04_FillWaterAuto | L211 | Bool | W | 1.0 | Tank04 Fill Water Auto |  |  |
| Tank04_FillWaterOn | L111 | Bool | W | 1.0 | Tank04 Fill Water always On |  |  |
| Tank04_FillWaterStatus | D1012.C | Bool | R | 1.0 | Tank04 Fill Water Valve Current Status |  |  |
| Tank04_HaveBasket | M604 | Bool | R | 1.0 | Tank04 The Tank has Basket |  |  |
| Tank04_RemoteStart | M331 | Bool | W | 1.0 | Tank04 Remote Start |  |  |
| Tank04_TimePV | CN104 | Word | R | 1.0 | Tank04 Time Present Value |  |  |
| Tank04_TimeSV | D4054 | Word | W | 1.0 | Tank04 Time Set Value | 0.0 | 9999.0 |
| Tank04_TimeUp | CS104 | Bool | R | 1.0 | Tank04 Time Up |  |  |
| Tank04_WaterFlow | D1016 | Word | R | 0.1 | Tank04 Water Flow Meter Value (LPM) |  |  |
| Tank05_DoorCloseRemote | M305 | Bool | W | 1.0 | Tank05 Door Remote Close |  |  |
| Tank05_DoorClosed | D1020.5 | Bool | R | 1.0 | Tank05 Door Actually Closed |  |  |
| Tank05_DoorOpenRemote | M304 | Bool | W | 1.0 | Tank05 Door Remote Open |  |  |
| Tank05_DoorOpened | D1020.4 | Bool | R | 1.0 | Tank05 Door Actually Opened |  |  |
| Tank05_FillWaterAuto | L220 | Bool | W | 1.0 | Tank05 Fill Water Auto |  |  |
| Tank05_FillWaterOn | L120 | Bool | W | 1.0 | Tank05 Fill Water always On |  |  |
| Tank05_FillWaterStatus | D1022.8 | Bool | R | 1.0 | Tank05 Fill Water Valve Current Status |  |  |
| Tank05_FilterRemote | M342 | Bool | W | 1.0 | Tank05 Filter Remote On |  |  |
| Tank05_FilterStatus | D1022.1 | Bool | R | 1.0 | Tank05 Filter Pump Status |  |  |
| Tank05_HaveBasket | M605 | Bool | R | 1.0 | Tank05 The Tank has Basket |  |  |
| Tank05_HeaterRemote | M343 | Bool | W | 1.0 | Tank05 Heater Remote On |  |  |
| Tank05_HeaterStatus | D1022.2 | Bool | R | 1.0 | Tank05 Heater Status |  |  |
| Tank05_RemoteStart | M340 | Bool | W | 1.0 | Tank05 Remote Start |  |  |
| Tank05_SwingRemote | M344 | Bool | W | 1.0 | Tank05 Swing Remote On |  |  |
| Tank05_SwingSV | D1018 | Word | W | 1.0 | Tank05 Swing Speed Set Value | 0.0 | 2000.0 |
| Tank05_SwingStatus | D1022.4 | Bool | R | 1.0 | Tank05 Swing Status |  |  |
| Tank05_TempAdj | D4012 | Word | W | 0.1 | Tank05 Temperature +- Adjust | 0.0 | 999.9 |
| Tank05_TempPV | D4002 | Word | R | 0.1 | Tank05 Temperature Present Value |  |  |
| Tank05_TempSV | D4022 | Word | W | 0.1 | Tank05 Temperature Set Value | 0.0 | 999.9 |
| Tank05_TimePV | CN105 | Word | R | 1.0 | Tank05 Time Present Value |  |  |
| Tank05_TimeSV | D4055 | Word | W | 1.0 | Tank05 Time Set Value | 0.0 | 9999.0 |
| Tank05_TimeUp | CS105 | Bool | R | 1.0 | Tank05 Time Up |  |  |
| Tank05_WaterFlow | D1024 | Float | R | 1.0 | Tank05 Water Flow Meter Value (LPM) |  |  |
| Tank06_AirAgitationAuto | L222 | Bool | W | 1.0 | Tank06 Air Agitation Auto |  |  |
| Tank06_AirAgitationOn | L122 | Bool | W | 1.0 | Tank06 Air Agitation Always On |  |  |
| Tank06_AirAgitationStatus | D1022.8 | Bool | R | 1.0 | Tank06 Air Agitation Status |  |  |
| Tank06_FillWaterAuto | L221 | Bool | W | 1.0 | Tank06 Fill Water Auto |  |  |
| Tank06_FillWaterOn | L121 | Bool | W | 1.0 | Tank06 Fill Water always On |  |  |
| Tank06_FillWaterStatus | D1022.C | Bool | R | 1.0 | Tank06 Fill Water Valve Current Status |  |  |
| Tank06_HaveBasket | M606 | Bool | R | 1.0 | Tank06 The Tank has Basket |  |  |
| Tank06_RemoteStart | M341 | Bool | W | 1.0 | Tank06 Remote Start |  |  |
| Tank06_TimePV | CN106 | Word | R | 1.0 | Tank06 Time Present Value |  |  |
| Tank06_TimeSV | D4056 | Word | W | 1.0 | Tank06 Time Set Value | 0.0 | 9999.0 |
| Tank06_TimeUp | CS106 | Bool | R | 1.0 | Tank06 Time Up |  |  |
| Tank06_WaterFlow | D1026 | Word | R | 0.1 | Tank06 Water Flow Meter Value (LPM) |  |  |
| Tank07_AirAgitationAuto | L235 | Bool | W | 1.0 | Tank07 Air Agitation Auto |  |  |
| Tank07_AirAgitationOn | L135 | Bool | W | 1.0 | Tank07 Air Agitation Always On |  |  |
| Tank07_AirAgitationStatus | D1030.F | Bool | R | 1.0 | Tank07 Air Agitation Status |  |  |
| Tank07_AirFlow | D1034 | Word | R | 0.1 | Tank07 Air Flow Meter Value |  |  |
| Tank07_AmperePV | D1055 | Word | R | 0.01 | Tank07 Ampere Present Value |  |  |
| Tank07_AmpereSV | D1038 | Word | W | 0.01 | Tank07 Ampere Set Value | 0.0 | 200.0 |
| Tank07_CoolerAuto | L231 | Bool | W | 1.0 | Tank07 Cool Water Circulation Auto |  |  |
| Tank07_CoolerOn | L131 | Bool | W | 1.0 | Tank07 Cool Water Circulation Always On |  |  |
| Tank07_CoolerStatus | D1030.B | Bool | R | 1.0 | Tank07 Cool Water Circulation Status |  |  |
| Tank07_DoorCloseRemote | M307 | Bool | W | 1.0 | Tank07 Door Remote Close |  |  |
| Tank07_DoorClosed | D1028.5 | Bool | R | 1.0 | Tank07 Door Actually Closed |  |  |
| Tank07_DoorOpenRemote | M306 | Bool | W | 1.0 | Tank07 Door Remote Open |  |  |
| Tank07_DoorOpened | D1028.4 | Bool | R | 1.0 | Tank07 Door Actually Opened |  |  |
| Tank07_FillWaterAuto | L230 | Bool | W | 1.0 | Tank07 Fill Water Auto |  |  |
| Tank07_FillWaterOn | L130 | Bool | W | 1.0 | Tank07 Fill Water always On |  |  |
| Tank07_FillWaterStatus | D1030.8 | Bool | R | 1.0 | Tank07 Fill Water Valve Current Status |  |  |
| Tank07_FilterRemote | M353 | Bool | W | 1.0 | Tank07 Filter Remote On |  |  |
| Tank07_FilterStatus | D1030.1 | Bool | R | 1.0 | Tank07 Filter Pump Status |  |  |
| Tank07_HaveBasket | M607 | Bool | R | 1.0 | Tank07 The Tank has Basket |  |  |
| Tank07_RectRemote | M354 | Bool | W | 1.0 | Tank07 Rectifier RemoteOn |  |  |
| Tank07_RectStatus | D1030.4 | Bool | R | 1.0 | Tank07 Rectifier Status |  |  |
| Tank07_RemoteStart | M350 | Bool | W | 1.0 | Tank07 Remote Start |  |  |
| Tank07_TempAdj | D4013 | Word | W | 0.1 | Tank07 Temperature +- Adjust | 0.0 | 999.9 |
| Tank07_TempPV | D4003 | Word | R | 0.1 | Tank07 Temperature Present Value |  |  |
| Tank07_TempSV | D4023 | Word | W | 0.1 | Tank07 Temperature Set Value | 0.0 | 999.9 |
| Tank07_TimePV | CN107 | Word | R | 1.0 | Tank07 Time Present Value |  |  |
| Tank07_TimeSV | D4057 | Word | W | 1.0 | Tank07 Time Set Value | 0.0 | 9999.0 |
| Tank07_TimeUp | CS107 | Bool | R | 1.0 | Tank07 Time Up |  |  |
| Tank07_VoltagePV | D1054 | Word | R | 0.01 | Tank07 Rectifier Voltage Present Value |  |  |
| Tank07_WaterFlow | D1032 | Float | R | 1.0 | Tank07 Water Flow Meter Value (LPM) |  |  |
| Tank08_AirAgitationAuto | L232 | Bool | W | 1.0 | Tank08 Air Agitation Auto |  |  |
| Tank08_AirAgitationOn | L132 | Bool | W | 1.0 | Tank08 Air Agitation Always On |  |  |
| Tank08_AirAgitationStatus | D1030.C | Bool | R | 1.0 | Tank08 Air Agitation Status |  |  |
| Tank08_HaveBasket | M608 | Bool | R | 1.0 | Tank08 The Tank has Basket |  |  |
| Tank08_RemoteStart | M351 | Bool | W | 1.0 | Tank08 Remote Start |  |  |
| Tank08_TimePV | CN108 | Word | R | 1.0 | Tank08 Time Present Value |  |  |
| Tank08_TimeSV | D4058 | Word | W | 1.0 | Tank08 Time Set Value | 0.0 | 9999.0 |
| Tank08_TimeUp | CS108 | Bool | R | 1.0 | Tank08 Time Up |  |  |
| Tank08_WaterFlow | D1036 | Word | R | 0.1 | Tank08 Water Flow Meter Value (LPM) |  |  |
| Tank09_AirAgitationAuto | L233 | Bool | W | 1.0 | Tank09 Air Agitation Auto |  |  |
| Tank09_AirAgitationOn | L133 | Bool | W | 1.0 | Tank09 Air Agitation Always On |  |  |
| Tank09_AirAgitationStatus | D1030.D | Bool | R | 1.0 | Tank09 Air Agitation Status |  |  |
| Tank09_FillWaterAuto | L234 | Bool | W | 1.0 | Tank09 Fill Water Auto |  |  |
| Tank09_FillWaterOn | L134 | Bool | W | 1.0 | Tank09 Fill Water always On |  |  |
| Tank09_FillWaterStatus | D1030.E | Bool | R | 1.0 | Tank09 Fill Water Valve Current Status |  |  |
| Tank09_HaveBasket | M609 | Bool | R | 1.0 | Tank09 The Tank has Basket |  |  |
| Tank09_RemoteStart | M352 | Bool | W | 1.0 | Tank09 Remote Start |  |  |
| Tank09_TimePV | CN109 | Word | R | 1.0 | Tank09 Time Present Value |  |  |
| Tank09_TimeSV | D4059 | Word | W | 1.0 | Tank09 Time Set Value | 0.0 | 9999.0 |
| Tank09_TimeUp | CS109 | Bool | R | 1.0 | Tank09 Time Up |  |  |
| Tank10_DoorCloseRemote | M309 | Bool | W | 1.0 | Tank10 Door Remote Close |  |  |
| Tank10_DoorClosed | D1102.5 | Bool | R | 1.0 | Tank10 Door Actually Closed |  |  |
| Tank10_DoorOpenRemote | M308 | Bool | W | 1.0 | Tank10 Door Remote Open |  |  |
| Tank10_DoorOpened | D1102.4 | Bool | R | 1.0 | Tank10 Door Actually Opened |  |  |
| Tank10_FillWaterAuto | L240 | Bool | W | 1.0 | Tank10 Fill Water Auto |  |  |
| Tank10_FillWaterOn | L140 | Bool | W | 1.0 | Tank10 Fill Water always On |  |  |
| Tank10_FillWaterStatus | D1104.8 | Bool | R | 1.0 | Tank10 Fill Water Valve Current Status |  |  |
| Tank10_FilterRemote | M361 | Bool | W | 1.0 | Tank10 Filter Remote On |  |  |
| Tank10_FilterStatus | D1104.1 | Bool | R | 1.0 | Tank10 Filter Pump Status |  |  |
| Tank10_HaveBasket | M610 | Bool | R | 1.0 | Tank10 The Tank has Basket |  |  |
| Tank10_HeaterRemote | M362 | Bool | W | 1.0 | Tank10 Heater Remote On |  |  |
| Tank10_HeaterStatus | D1104.2 | Bool | R | 1.0 | Tank10 Heater Status |  |  |
| Tank10_PH | D1108 | Word | R | 0.01 | Tank10 PH Meter Value |  |  |
| Tank10_RemoteStart | M360 | Bool | W | 1.0 | Tank10 Remote Start |  |  |
| Tank10_SwingRemote | M363 | Bool | W | 1.0 | Tank10 Swing Remote On |  |  |
| Tank10_SwingSV | D1100 | Word | W | 1.0 | Tank10 Swing Speed Set Value | 0.0 | 2000.0 |
| Tank10_SwingStatus | D1104.3 | Bool | R | 1.0 | Tank10 Swing Status |  |  |
| Tank10_TempAdj | D4014 | Word | W | 0.1 | Tank10 Temperature +- Adjust | 0.0 | 999.9 |
| Tank10_TempPV | D4004 | Word | R | 0.1 | Tank10 Temperature Present Value |  |  |
| Tank10_TempSV | D4024 | Word | W | 0.1 | Tank10 Temperature Set Value | 0.0 | 999.9 |
| Tank10_TimePV | CN110 | Word | R | 1.0 | Tank10 Time Present Value |  |  |
| Tank10_TimeSV | D4060 | Word | W | 1.0 | Tank10 Time Set Value | 0.0 | 9999.0 |
| Tank10_TimeUp | CS110 | Bool | R | 1.0 | Tank10 Time Up |  |  |
| Tank10_WaterFlow | D1106 | Float | R | 1.0 | Tank10 Water Flow Meter Value (LPM) |  |  |
| Tank11_FillWaterAuto | L250 | Bool | W | 1.0 | Tank11 Fill Water Auto |  |  |
| Tank11_FillWaterOn | L150 | Bool | W | 1.0 | Tank11 Fill Water always On |  |  |
| Tank11_FillWaterStatus | D1112.4 | Bool | R | 1.0 | Tank11 Fill Water Valve Current Status |  |  |
| Tank11_HaveBasket | M611 | Bool | R | 1.0 | Tank11 The Tank has Basket |  |  |
| Tank11_RemoteStart | M370 | Bool | W | 1.0 | Tank11 Remote Start |  |  |
| Tank11_SprayRemote | M371 | Bool | W | 1.0 | Tank11 Spray Remote On |  |  |
| Tank11_SprayStatus | D1112.1 | Bool | R | 1.0 | Tank11 Spray Status |  |  |
| Tank11_TimePV | CN111 | Word | R | 1.0 | Tank11 Time Present Value |  |  |
| Tank11_TimeSV | D4061 | Word | W | 1.0 | Tank11 Time Set Value | 0.0 | 9999.0 |
| Tank11_TimeUp | CS111 | Bool | R | 1.0 | Tank11 Time Up |  |  |
| Tank11_WaterFlow | D1114 | Word | R | 0.1 | Tank11 Water Flow Meter Value (LPM) |  |  |
| Tank12_AirAgitationAuto | L261 | Bool | W | 1.0 | Tank12 Air Agitation Auto |  |  |
| Tank12_AirAgitationOn | L161 | Bool | W | 1.0 | Tank12 Air Agitation Always On |  |  |
| Tank12_AirAgitationStatus | D1118.4 | Bool | R | 1.0 | Tank12 Air Agitation Status |  |  |
| Tank12_DoorCloseRemote | M311 | Bool | W | 1.0 | Tank12 Door Remote Close |  |  |
| Tank12_DoorClosed | D1116.5 | Bool | R | 1.0 | Tank12 Door Actually Closed |  |  |
| Tank12_DoorOpenRemote | M310 | Bool | W | 1.0 | Tank12 Door Remote Open |  |  |
| Tank12_DoorOpened | D1116.4 | Bool | R | 1.0 | Tank12 Door Actually Opened |  |  |
| Tank12_FillWaterAuto | L262 | Bool | W | 1.0 | Tank12 Fill Water Auto |  |  |
| Tank12_FillWaterOn | L162 | Bool | W | 1.0 | Tank12 Fill Water always On |  |  |
| Tank12_FillWaterStatus | D1118.5 | Bool | R | 1.0 | Tank12 Fill Water Valve Current Status |  |  |
| Tank12_HaveBasket | M612 | Bool | R | 1.0 | Tank12 The Tank has Basket |  |  |
| Tank12_RemoteStart | M380 | Bool | W | 1.0 | Tank12 Remote Start |  |  |
| Tank12_TimePV | CN112 | Word | R | 1.0 | Tank12 Time Present Value |  |  |
| Tank12_TimeSV | D4062 | Word | W | 1.0 | Tank12 Time Set Value | 0.0 | 9999.0 |
| Tank12_TimeUp | CS112 | Bool | R | 1.0 | Tank12 Time Up |  |  |
| Tank12_WaterFlow | D1120 | Word | R | 0.1 | Tank12 Water Flow Meter Value (LPM) |  |  |
| Tank13_Conductivity | D1122 | Float | R | 1.0 | Tank13 Conductivity Meter Reading |  |  |
| Tank13_FillWaterAuto | L260 | Bool | W | 1.0 | Tank13 Fill Water Auto |  |  |
| Tank13_FillWaterOn | L160 | Bool | W | 1.0 | Tank13 Fill Water always On |  |  |
| Tank13_FillWaterStatus | D1118.3 | Bool | R | 1.0 | Tank13 Fill Water Valve Current Status |  |  |
| Tank13_HaveBasket | M613 | Bool | R | 1.0 | Tank13 The Tank has Basket |  |  |
| Tank13_HeaterRemote | M382 | Bool | W | 1.0 | Tank13 Heater Remote On |  |  |
| Tank13_HeaterStatus | D1118.2 | Bool | R | 1.0 | Tank13 Heater Status |  |  |
| Tank13_RemoteStart | M381 | Bool | W | 1.0 | Tank13 Remote Start |  |  |
| Tank13_TempAdj | D4015 | Word | W | 0.1 | Tank13 Temperature +- Adjust | 0.0 | 999.9 |
| Tank13_TempPV | D4005 | Word | R | 0.1 | Tank13 Temperature Present Value |  |  |
| Tank13_TempSV | D4025 | Word | W | 0.1 | Tank13 Temperature Set Value | 0.0 | 999.9 |
| Tank13_TimePV | CN113 | Word | R | 1.0 | Tank13 Time Present Value |  |  |
| Tank13_TimeSV | D4063 | Word | W | 1.0 | Tank13 Time Set Value | 0.0 | 9999.0 |
| Tank13_TimeUp | CS113 | Bool | R | 1.0 | Tank13 Time Up |  |  |
| Tank14_DoorCloseRemote | M313 | Bool | W | 1.0 | Tank14 Door Remote Close |  |  |
| Tank14_DoorClosed | D1126.5 | Bool | R | 1.0 | Tank14 Door Actually Closed |  |  |
| Tank14_DoorOpenRemote | M312 | Bool | W | 1.0 | Tank14 Door Remote Open |  |  |
| Tank14_DoorOpened | D1126.4 | Bool | R | 1.0 | Tank14 Door Actually Opened |  |  |
| Tank14_FillWaterAuto | L270 | Bool | W | 1.0 | Tank14 Fill Water Auto |  |  |
| Tank14_FillWaterOn | L170 | Bool | W | 1.0 | Tank14 Fill Water always On |  |  |
| Tank14_FillWaterStatus | D1128.8 | Bool | R | 1.0 | Tank14 Fill Water Valve Current Status |  |  |
| Tank14_FilterRemote | M391 | Bool | W | 1.0 | Tank14 Filter Remote On |  |  |
| Tank14_FilterStatus | D1128.1 | Bool | R | 1.0 | Tank14 Filter Pump Status |  |  |
| Tank14_HaveBasket | M614 | Bool | R | 1.0 | Tank14 The Tank has Basket |  |  |
| Tank14_HeaterRemote | M392 | Bool | W | 1.0 | Tank14 Heater Remote On |  |  |
| Tank14_HeaterStatus | D1128.2 | Bool | R | 1.0 | Tank14 Heater Status |  |  |
| Tank14_PH | D1132 | Word | R | 0.01 | Tank14 PH Meter Value |  |  |
| Tank14_RemoteStart | M390 | Bool | W | 1.0 | Tank14 Remote Start |  |  |
| Tank14_SwingRemote | M393 | Bool | W | 1.0 | Tank14 Swing Remote On |  |  |
| Tank14_SwingSV | D1124 | Word | W | 1.0 | Tank14 Swing Speed Set Value | 0.0 | 2000.0 |
| Tank14_SwingStatus | D1128.3 | Bool | R | 1.0 | Tank14 Swing Status |  |  |
| Tank14_TempAdj | D4016 | Word | W | 0.1 | Tank14 Temperature +- Adjust | 0.0 | 999.9 |
| Tank14_TempPV | D4006 | Word | R | 0.1 | Tank14 Temperature Present Value |  |  |
| Tank14_TempSV | D4026 | Word | W | 0.1 | Tank14 Temperature Set Value | 0.0 | 999.9 |
| Tank14_TimePV | CN114 | Word | R | 1.0 | Tank14 Time Present Value |  |  |
| Tank14_TimeSV | D4064 | Word | W | 1.0 | Tank14 Time Set Value | 0.0 | 9999.0 |
| Tank14_TimeUp | CS114 | Bool | R | 1.0 | Tank14 Time Up |  |  |
| Tank14_WaterFlow | D1130 | Float | R | 1.0 | Tank14 Water Flow Meter Value (LPM) |  |  |
| Tank15_AirAgitationAuto | L280 | Bool | W | 1.0 | Tank15 Air Agitation Auto |  |  |
| Tank15_AirAgitationOn | L180 | Bool | W | 1.0 | Tank15 Air Agitation Always On |  |  |
| Tank15_AirAgitationStatus | D1136.4 | Bool | R | 1.0 | Tank15 Air Agitation Status |  |  |
| Tank15_FillWaterAuto | L281 | Bool | W | 1.0 | Tank15 Fill Water Auto |  |  |
| Tank15_FillWaterOn | L181 | Bool | W | 1.0 | Tank15 Fill Water always On |  |  |
| Tank15_FillWaterStatus | D1136.5 | Bool | R | 1.0 | Tank15 Fill Water Valve Current Status |  |  |
| Tank15_HaveBasket | M615 | Bool | R | 1.0 | Tank15 The Tank has Basket |  |  |
| Tank15_RemoteStart | M400 | Bool | W | 1.0 | Tank15 Remote Start |  |  |
| Tank15_TimePV | CN115 | Word | R | 1.0 | Tank15 Time Present Value |  |  |
| Tank15_TimeSV | D4065 | Word | W | 1.0 | Tank15 Time Set Value | 0.0 | 9999.0 |
| Tank15_TimeUp | CS115 | Bool | R | 1.0 | Tank15 Time Up |  |  |
| Tank15_WaterFlow | D1138 | Word | R | 0.1 | Tank15 Water Flow Meter Value (LPM) |  |  |
| Tank16_AirAgitationAuto | L282 | Bool | W | 1.0 | Tank16 Air Agitation Auto |  |  |
| Tank16_AirAgitationOn | L182 | Bool | W | 1.0 | Tank16 Air Agitation Always On |  |  |
| Tank16_AirAgitationStatus | D1136.6 | Bool | R | 1.0 | Tank16 Air Agitation Status |  |  |
| Tank16_FillWaterAuto | L283 | Bool | W | 1.0 | Tank16 Fill Water Auto |  |  |
| Tank16_FillWaterOn | L183 | Bool | W | 1.0 | Tank16 Fill Water always On |  |  |
| Tank16_FillWaterStatus | D1136.7 | Bool | R | 1.0 | Tank16 Fill Water Valve Current Status |  |  |
| Tank16_FilterRemote | M402 | Bool | W | 1.0 | Tank16 Filter Remote On |  |  |
| Tank16_FilterStatus | D1136.2 | Bool | R | 1.0 | Tank16 Filter Pump Status |  |  |
| Tank16_HaveBasket | M616 | Bool | R | 1.0 | Tank16 The Tank has Basket |  |  |
| Tank16_HeaterRemote | M403 | Bool | W | 1.0 | Tank16 Heater Remote On |  |  |
| Tank16_HeaterStatus | D1136.3 | Bool | R | 1.0 | Tank16 Heater Status |  |  |
| Tank16_PH | D1142 | Word | R | 0.01 | Tank16 PH Meter Value |  |  |
| Tank16_RemoteStart | M401 | Bool | W | 1.0 | Tank16 Remote Start |  |  |
| Tank16_TempAdj | D4017 | Word | W | 0.1 | Tank16 Temperature +- Adjust | 0.0 | 999.9 |
| Tank16_TempPV | D4007 | Word | R | 0.1 | Tank16 Temperature Present Value |  |  |
| Tank16_TempSV | D4027 | Word | W | 0.1 | Tank16 Temperature Set Value | 0.0 | 999.9 |
| Tank16_TimePV | CN116 | Word | R | 1.0 | Tank16 Time Present Value |  |  |
| Tank16_TimeSV | D4066 | Word | W | 1.0 | Tank16 Time Set Value | 0.0 | 9999.0 |
| Tank16_TimeUp | CS116 | Bool | R | 1.0 | Tank16 Time Up |  |  |
| Tank16_WaterFlow | D1140 | Word | R | 0.1 | Tank16 Water Flow Meter Value (LPM) |  |  |

# 9. Client Examples

## 9.1 curl

curl -H "X-API-Key: YOUR_KEY" http://127.0.0.1:8000/api/v1/tags/list

curl -H "X-API-Key: YOUR_KEY" http://127.0.0.1:8000/api/v1/tags/read

curl -H "X-API-Key: YOUR_KEY" -H "Content-Type: application/json" \
  -d "{\"tag_name\":\"Tank01_TempSV\",\"value\":25.0}" \
  http://127.0.0.1:8000/api/v1/tags/write

## 9.2 Python

import json
import urllib.request

key = open("dev_api_key.txt").read().strip()
req = urllib.request.Request(
    "http://127.0.0.1:8000/api/v1/tags/read",
    headers={"X-API-Key": key},
)
with urllib.request.urlopen(req) as resp:
    print(json.loads(resp.read()))

# 10. Error Codes

| Status | Typical cause |
| --- | --- |
| 400 | Invalid value, out of Low/High limit, unsupported type, duplicate batch tags |
| 401 | Missing/invalid X-API-Key |
| 403 | Tag is read-only (ReadWrite=R) |
| 404 | Tag not found, or no poll records yet |
| 502 | PLC write/communication failure |
| 503 | API key missing, database missing, PLC busy/unavailable |

# 11. Troubleshooting

| Problem | What to do |
| --- | --- |
| 503 API key not configured | Run python generate_api_key.py |
| 503 Database not found | Run plc_polling.py or set PLC_DB_PATH |
| 503 PLC is busy | Retry after the current poll cycle finishes |
| 401 Invalid API key | Match X-API-Key to dev_api_key.txt / PLC_API_KEY |
| 404 No polling records | Start the poller and wait for one cycle |
| 400 below/above limit | Check /tags/list low_limit and high_limit |
| Port already in use | Set PLC_API_PORT to another port |

End of PLC API User Manual